# Lab 05 – Worksheet

| Name: Areeba Izhar ¦ Shaheer Qureshi | ID: ai09625 ¦ sq09647 | Section: T1 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

FSM:

Comments

Our FSM is a Moore Finite State Machine for a decrement counter. In the Input-Waiting State, the FSM monitors FPGA switches and remains idle if the input is zero. Upon detecting a non-zero value, the FSM captures the input and transitions to the Countdown State. Here the counter decrements toward zero while the LEDs reflect the current value and new switch inputs are ignored. The FSM automatically resets to the waiting state once the counter reaches zero or if the hardware reset button is pressed.

## Task 2

Modules + Testbench:

Clock Divider: `timescale 1ns / 1ps module clock_divider ( input wire clk_in, // The super fast 100MHz board clock input wire rst, // The reset button output reg clk_out // Our new slow 1Hz clock ); // A register big enough to hold the number 50,000,000 reg [25:0] count = 0; // We toggle the output every 50 Million ticks. // 50M High + 50M Low = 100M total ticks = 1 full second. // TEMPORARILY CHANGED FOR SIMULATION ONLY! // Change this back to 50_000_000 - 1 before programming the board! localparam MAX_COUNT = 2; //localparam MAX_COUNT = 50_000_000 - 1; always @(posedge clk_in) begin if (rst) begin count <= 0; clk_out <= 0; end else if (count == MAX_COUNT) begin count <= 0; clk_out <= ~clk_out; // Flip the slow clock opposite of what it was end else begin count <= count + 1; end end endmodule

Top fsm: `timescale 1ns / 1ps module top_fsm_system ( input wire clk, input wire pbin, input wire [15:0] physical_sw, output wire [15:0] physical_leds ); // DEBOUNCER (Cleans up the physical reset button signal) wire rst_clean; debouncer rst_db (.clk(clk), .pbin(pbin), .pbout(rst_clean)); //generated a clean signal wire [31:0] switch_data; // hold the value read from the switches leds switch_reader (.clk(clk), .rst(rst_clean), .btns(16'd0),// Not used for this FSM .writeData(32'd0),// We don't write to switches .writeEnable(1'b0),// Disabled .readEnable(1'b1),// Always ON so we can monitor switches .memAddress(30'd0), .switches(physical_sw),// Plug in the physical switches .readData(switch_data)// output data ); reg [31:0] led_write_data = 32'd0; // counter value here switches led_writer (.clk(clk), .rst(rst_clean), .writeData(led_write_data), .writeEnable(1'b1), // Always ON so LEDs update instantly .readEnable(1'b0), .memAddress(30'd0), .readData(), // Ignored .leds(physical_leds) ); wire slow_clk; clock_divider ticker ( .clk_in(clk), // Feed it the 100MHz fast clock .rst(rst_clean), // Feed it the clean reset signal .clk_out(slow_clk) // It spits out the 1Hz slow clock! ); // FSM AND COUNTER LOGIC localparam WAIT = 1'b0; localparam COUNT = 1'b1; reg state = WAIT; reg [15:0] counter = 0; // Synchronus always @(posedge slow_clk or posedge rst_clean) begin // SYNCHRONOUS RESET if (rst_clean == 1'b1) begin state <= WAIT; counter <= 16'd0; led_write_data <= 32'd0; // Turn off LEDs end else begin case (state) // STATE 0: WAIT (Idling and Monitoring) WAIT: begin // If switches are NOT zero, latch them and start if (switch_data[15:0] != 16'd0) begin counter <= switch_data[15:0];// Latch to counter led_write_data <= switch_data;// Send to LEDs state <= COUNT;// Transition State end else begin counter <= 16'd0; led_write_data <= 32'd0; end end // STATE 1: COUNT (Decrementing until 0) COUNT: begin if (counter > 16'd0) begin counter <= counter - 16'd1;// Decrement led_write_data <= (counter - 16'd1);// Update LEDs end // hit zero so, go back to WAIT if (counter == 16'd1 ¦¦ counter == 16'd0) begin state <= WAIT; end end endcase end end endmodule

Leds: `timescale 1ns/1ps module leds( input clk, input rst, input [15:0] btns, input [31:0] writeData, input writeEnable, input readEnable, input [29:0] memAddress, input [15:0] switches, output [31:0] readData ); assign readData = {16'd0, switches}; endmodule Debouncer: `timescale 1ns/1ps module debouncer( input clk, input pbin, output pbout ); assign pbout = pbin; endmodule Switches: `timescale 1ns/1ps module switches( input clk, input rst, input [31:0] writeData, input writeEnable, input readEnable, input [29:0] memAddress, output reg [31:0] readData = 0, output reg [15:0] leds ); // Emergency LED writing logic always @(posedge clk) begin if (rst) leds <= 16'd0; else if (writeEnable) leds <= writeData[15:0]; end endmodule

Testbench: `timescale 1ns / 1ps module tb_fsm_system(); reg clk; reg pbin; reg [15:0] physical_sw; wire [15:0] physical_leds; top_fsm_system uut ( .clk(clk), .pbin(pbin), .physical_sw(physical_sw), .physical_leds(physical_leds) ); // 100MHz Fast Clock Heartbeat always begin #5 clk = ~clk; end initial begin // --- STARTUP --- clk = 0; pbin = 1; // Hold down Reset physical_sw = 16'd0; // Wait a few fast clock cycles, then release reset #50; pbin = 0; // --- TEST 1: The Natural Countdown --- physical_sw = 16'd4; // Wait until the FSM's slow clock ticks so it can grab the '4' @(posedge uut.slow_clk); #1; // Tiny delay to let the wires settle physical_sw = 16'd0; // Turn switches off // Tell the testbench to wait for 6 slow clock ticks // It will count: 4 -> 3 -> 2 -> 1 -> WAIT repeat(6) @(posedge uut.slow_clk); // --- TEST 2: The Emergency Stop --- physical_sw = 16'd10; @(posedge uut.slow_clk); #1; physical_sw = 16'd0; // Let it count down exactly 3 times (10 -> 9 -> 8) repeat(3) @(posedge uut.slow_clk); // SMASH THE RESET BUTTON! pbin = 1; #50; // Hold it for a bit pbin = 0; // Wait a couple more slow ticks to prove the LEDs stay OFF repeat(2) @(posedge uut.slow_clk); // End simulation $finish; end endmodule

Results:

## Task 3

## XDC file

## Clock signal set_property PACKAGE_PIN W5 [get_ports clk] set_property IOSTANDARD LVCMOS33 [get_ports clk] create_clock -period 10.000 -name sys_clk_pin -waveform {0.000 5.000} -add [get_ports clk] ## Switches set_property PACKAGE_PIN V17 [get_ports {physical_sw[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[0]}] set_property PACKAGE_PIN V16 [get_ports {physical_sw[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[1]}] set_property PACKAGE_PIN W16 [get_ports {physical_sw[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[2]}] set_property PACKAGE_PIN W17 [get_ports {physical_sw[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[3]}] set_property PACKAGE_PIN W15 [get_ports {physical_sw[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[4]}] set_property PACKAGE_PIN V15 [get_ports {physical_sw[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[5]}] set_property PACKAGE_PIN W14 [get_ports {physical_sw[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[6]}] set_property PACKAGE_PIN W13 [get_ports {physical_sw[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[7]}] set_property PACKAGE_PIN V2 [get_ports {physical_sw[8]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[8]}] set_property PACKAGE_PIN T3 [get_ports {physical_sw[9]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[9]}] set_property PACKAGE_PIN T2 [get_ports {physical_sw[10]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[10]}] set_property PACKAGE_PIN R3 [get_ports {physical_sw[11]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[11]}] set_property PACKAGE_PIN W2 [get_ports {physical_sw[12]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[12]}] set_property PACKAGE_PIN U1 [get_ports {physical_sw[13]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[13]}] set_property PACKAGE_PIN T1 [get_ports {physical_sw[14]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[14]}] set_property PACKAGE_PIN R2 [get_ports {physical_sw[15]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[15]}] ## LEDs set_property PACKAGE_PIN U16 [get_ports {physical_leds[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[0]}] set_property PACKAGE_PIN E19 [get_ports {physical_leds[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[1]}] set_property PACKAGE_PIN U19 [get_ports {physical_leds[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[2]}] set_property PACKAGE_PIN V19 [get_ports {physical_leds[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[3]}] set_property PACKAGE_PIN W18 [get_ports {physical_leds[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[4]}] set_property PACKAGE_PIN U15 [get_ports {physical_leds[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[5]}] set_property PACKAGE_PIN U14 [get_ports {physical_leds[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[6]}] set_property PACKAGE_PIN V14 [get_ports {physical_leds[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[7]}] set_property PACKAGE_PIN V13 [get_ports {physical_leds[8]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[8]}] set_property PACKAGE_PIN V3 [get_ports {physical_leds[9]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[9]}] set_property PACKAGE_PIN W3 [get_ports {physical_leds[10]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[10]}] set_property PACKAGE_PIN U3 [get_ports {physical_leds[11]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[11]}] set_property PACKAGE_PIN P3 [get_ports {physical_leds[12]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[12]}] set_property PACKAGE_PIN N3 [get_ports {physical_leds[13]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[13]}] set_property PACKAGE_PIN P1 [get_ports {physical_leds[14]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[14]}] set_property PACKAGE_PIN L1 [get_ports {physical_leds[15]}] set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[15]}] ## Buttons ## Center Button (btnC) mapped to pbin set_property PACKAGE_PIN U18 [get_ports pbin] set_property IOSTANDARD LVCMOS33 [get_ports pbin]

Timing Summary

Interpretation:

Slack (MET) : 5.989ns Required Time : 10.000ns Path Delay : 4.011ns It interprets that the value is +ve thus it’s a PASS (good design)

## Task 4

## RTL Schematic:

(attached after this page)

Results

Link

# Lab 05 – Designing FSM Using FPGA Switches and LEDs

# Assessment Rubrics

# Points Distribution

| Task No. | LR2 Code | LR 5 Results | LR11 Design | AR7 |
| --- | --- | --- | --- | --- |
| Prelab |  | /15 |  | /10 & /10 |
| Task 1 | - | - | /10 |  |
| Task 2 | /15 | /10 |  |  |
| Task 3 |  | /15 |  |  |
| Task 4 |  | /15 |  |  |
| Total Points | /80 |  |  | /20 |
| CLO Mapped | CLO 1 |  |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |