# Lab 7: Design and FPGA Implementation of the Register File for a Single-Cycle RISC-V Processor

Objective

# Task 1: Register File Block Diagram Design

# Block diagram of Register File

# Task 2: Verilog Description and Testbench Development

## Verilog source file for Register File 
`timescale 1ns / 1ps

## module RegisterFile (

## input  wire clk,

## input  wire rst,

## input  wire WriteEnable,

## input  wire [4:0]  rs1,   // reg 1

## input  wire [4:0]  rs2, // reg 2

## input  wire [4:0]  rd,    // destination reg

## input  wire [31:0] WriteData,

## output wire [31:0] ReadData1, //reg 1 data

## output wire [31:0] ReadData2 //reg 2 data

## );

## reg [31:0] regs [31:0]; //32 registers, each 32 bits wide

## integer k; // value 32 for loop

## always @(posedge clk or posedge rst) begin

## if (rst) begin

## //reset clears reg

## for (k = 0; k < 32; k = k + 1)

## regs[k] <= 32'd0;

## end else begin

## if (WriteEnable && (rd != 5'd0))

## regs[rd] <= WriteData;

## end

## end

## assign ReadData1 = (rs1 == 5'd0) ? 32'd0 : regs[rs1];

## assign ReadData2 = (rs2 == 5'd0) ? 32'd0 : regs[rs2];

## endmodule

## Testbench file for Register File
`timescale 1ns / 1ps

## module RegisterFile_tb;

## reg        clk;

## reg        rst;

## reg        WriteEnable;

## reg  [4:0] rs1, rs2, rd;

## reg  [31:0] WriteData;

## wire [31:0] ReadData1, ReadData2;

## RegisterFile uut (

## .clk(clk),

## .rst(rst),

## .WriteEnable(WriteEnable),

## .rs1(rs1),

## .rs2(rs2),

## .rd(rd),

## .WriteData(WriteData),

## .ReadData1(ReadData1),

## .ReadData2(ReadData2)

## );

## // 100 MHz clock

## always #5 clk = ~clk;

## task write_reg;

## input [4:0]  addr;

## input [31:0] data;

## begin

## @(negedge clk);           // set up before rising edge

## WriteEnable = 1;

## rd          = addr;

## WriteData   = data;

## @(posedge clk); #1;       // capture on rising edge

## WriteEnable = 0;

## end

## endtask

## integer pass_count = 0;

## integer fail_count = 0;

## initial begin

## // --- Initialise ---

## clk         = 0;

## rst         = 1;

## WriteEnable = 0;

## rs1         = 0;

## rs2         = 0;

## rd          = 0;

## WriteData   = 0;

## // Release reset after 3 clock cycles

## repeat (3) @(posedge clk);

## rst = 0;

## #1;

## write_reg(5'd5, 32'hDEAD_BEEF);

## rs1 = 5'd5;

## #1;  // asynchronous read settles immediately

## if (ReadData1 === 32'hDEAD_BEEF) begin

## $display("[PASS] Test i : x5 = 0x%08h (correct)", ReadData1);

## pass_count = pass_count + 1;

## end else begin

## $display("[FAIL] Test i : x5 expected 0xDEADBEEF, got 0x%08h", ReadData1);

## fail_count = fail_count + 1;

## end

## write_reg(5'd0, 32'hFFFF_FFFF);

## rs1 = 5'd0;

## #1;

## if (ReadData1 === 32'd0) begin

## $display("[PASS] Test ii: x0 = 0x%08h (correctly stays zero)", ReadData1);

## pass_count = pass_count + 1;

## end else begin

## $display("[FAIL] Test ii: x0 expected 0x00000000, got 0x%08h", ReadData1);

## fail_count = fail_count + 1;

## end

## write_reg(5'd10, 32'hCAFE_BABE);

## rs1 = 5'd5;   // should return 0xDEADBEEF

## rs2 = 5'd10;  // should return 0xCAFEBABE

## #1;

## if (ReadData1 === 32'hDEAD_BEEF && ReadData2 === 32'hCAFE_BABE) begin

## $display("[PASS] Test iii: Dual read OK  rs1=0x%08h  rs2=0x%08h", ReadData1, ReadData2);

## pass_count = pass_count + 1;

## end else begin

## $display("[FAIL] Test iii: rs1=0x%08h (exp 0xDEADBEEF)  rs2=0x%08h (exp 0xCAFEBABE)",

## ReadData1, ReadData2);

## fail_count = fail_count + 1;

## end

## write_reg(5'd5, 32'h1234_5678);

## rs1 = 5'd5;

## #1;

## if (ReadData1 === 32'h1234_5678) begin

## $display("[PASS] Test iv : x5 overwritten to 0x%08h (correct)", ReadData1);

## pass_count = pass_count + 1;

## end else begin

## $display("[FAIL] Test iv : x5 expected 0x12345678, got 0x%08h", ReadData1);

## fail_count = fail_count + 1;

## end

## rst = 1;

## @(posedge clk); #1;

## rst = 0;

## rs1 = 5'd5;

## rs2 = 5'd10;

## #1;

## if (ReadData1 === 32'd0 && ReadData2 === 32'd0) begin

## $display("[PASS] Test v  : Reset OK â€“ x5=0x%08h, x10=0x%08h", ReadData1, ReadData2);

## pass_count = pass_count + 1;

## end else begin

## $display("[FAIL] Test v  : After reset x5=0x%08h, x10=0x%08h (expected 0)", ReadData1, ReadData2);

## fail_count = fail_count + 1;

## end

## //

## $display("--------------------------------------------------");

## $display("RegisterFile_tb: %0d PASSED, %0d FAILED", pass_count, fail_count);

## $display("--------------------------------------------------");

## $finish;

## end

## endmodule

## Verilog source file for Top-level (top rf alu.v).
`timescale 1ns / 1ps

## module top_rf_alu (

## input  wire        clk,

## input  wire        pbin,

## input  wire [15:0] physical_sw,

## output wire [15:0] physical_leds

## );

## wire rst_clean;

## debouncer rst_db (

## .clk  (clk),

## .pbin (pbin),

## .pbout(rst_clean)

## );

## wire [4:0]  fsm_rs1, fsm_rs2, fsm_rd;

## wire [31:0] fsm_wd;

## wire        fsm_we;

## wire [3:0]  fsm_alu_ctrl;

## wire [3:0]  fsm_state_out;

## wire [31:0] rf_rd1, rf_rd2;

## wire [31:0] alu_result;

## wire        alu_zero;

## //

## RegisterFile RF (

## .clk        (clk),

## .rst        (rst_clean),

## .WriteEnable(fsm_we),

## .rs1        (fsm_rs1),

## .rs2        (fsm_rs2),

## .rd         (fsm_rd),

## .WriteData  (fsm_wd),

## .ReadData1  (rf_rd1),

## .ReadData2  (rf_rd2)

## );

## alu_32bit ALU (

## .A         (rf_rd1),

## .B         (rf_rd2),

## .ALUControl(fsm_alu_ctrl),

## .ALUResult (alu_result),

## .Zero      (alu_zero)

## );

## rf_alu_fsm FSM (

## .clk       (clk),

## .rst       (rst_clean),

## .alu_result(alu_result),

## .alu_zero  (alu_zero),

## .rf_we     (fsm_we),

## .rf_rs1    (fsm_rs1),

## .rf_rs2    (fsm_rs2),

## .rf_rd     (fsm_rd),

## .rf_wd     (fsm_wd),

## .alu_ctrl  (fsm_alu_ctrl),

## .state_out (fsm_state_out)

## );

## switches led_writer (

## .clk        (clk),

## .rst        (rst_clean),

## .writeData  ({16'd0, alu_zero, fsm_state_out, alu_result[10:0]}),

## .writeEnable(1'b1),

## .readEnable (1'b0),

## .memAddress (30'd0),

## .readData   (),

## .leds       (physical_leds)

## );

## endmodule

## module rf_alu_fsm (

## input  wire        clk,

## input  wire        rst,

## input  wire [31:0] alu_result,

## input  wire        alu_zero,

## // Register File control

## output reg         rf_we,

## output reg  [4:0]  rf_rs1,

## output reg  [4:0]  rf_rs2,

## output reg  [4:0]  rf_rd,

## output reg  [31:0] rf_wd,

## // ALU control

## output reg  [3:0]  alu_ctrl,

## // Status (for LED display)

## output reg  [3:0]  state_out

## );

## // ALU control codes - match Lab 6 alu_32bit

## localparam [3:0]

## CTRL_AND = 4'b0000,

## CTRL_OR  = 4'b0001,

## CTRL_ADD = 4'b0010,

## CTRL_SUB = 4'b0110,

## CTRL_XOR = 4'b0100,

## CTRL_SLL = 4'b1000,

## CTRL_SRL = 4'b1001;

## localparam [3:0]

## S_INIT0    = 4'd0,   // write x1 = 0x10101010

## S_INIT1    = 4'd1,   // write x2 = 0x01010101

## S_INIT2    = 4'd2,   // write x3 = 5 (shift amount)

## S_READ_ALU = 4'd3,   // present rs1=x1, rs2=x2, wait for ALU to settle

## S_WR_ADD   = 4'd4,   // write ADD result -> x4

## S_WR_SUB   = 4'd5,   // write SUB result -> x5

## S_WR_AND   = 4'd6,   // write AND result -> x6

## S_WR_OR    = 4'd7,   // write OR  result -> x7

## S_WR_XOR   = 4'd8,   // write XOR result -> x8

## S_READ_SLL = 4'd9,   // present rs1=x1, rs2=x3 for shift ops

## S_WR_SLL   = 4'd10,  // write SLL result -> x9

## S_WR_SRL   = 4'd11,  // write SRL result -> x10

## S_BEQ_READ = 4'd12,  // SUB x1-x1, check Zero flag

## S_BEQ_WR   = 4'd13,  // write flag -> x11

## S_DONE     = 4'd14;  // stable - hold results, wait for reset

## reg [3:0] state;

## // Track which op we're writing so we can change alu_ctrl in the WRITE state

## // without losing the correct result

## reg [3:0] saved_ctrl;

## always @(*) state_out = state[3:0];

## always @(posedge clk or posedge rst) begin

## if (rst) begin

## state    <= S_INIT0;

## rf_we    <= 0;

## rf_rs1   <= 0; rf_rs2 <= 0;

## rf_rd    <= 0; rf_wd  <= 0;

## alu_ctrl <= CTRL_ADD;

## saved_ctrl <= CTRL_ADD;

## end else begin

## // Default: no write

## rf_we <= 0;

## case (state)

## S_INIT0: begin

## rf_we <= 1; rf_rd <= 5'd1; rf_wd <= 32'h10101010;

## state <= S_INIT1;

## end

## S_INIT1: begin

## rf_we <= 1; rf_rd <= 5'd2; rf_wd <= 32'h01010101;

## state <= S_INIT2;

## end

## S_INIT2: begin

## rf_we <= 1; rf_rd <= 5'd3; rf_wd <= 32'd5;

## // Point rs1/rs2 at x1,x2 and set first op ready for next cycle

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd2; alu_ctrl <= CTRL_ADD;

## state <= S_READ_ALU;

## end

## S_READ_ALU: begin

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd2;

## // alu_ctrl is already ADD (set in INIT2 or carried forward)

## state <= S_WR_ADD;

## end

## // Write ADD result (alu_result is now ADD of x1,x2)

## S_WR_ADD: begin

## rf_we <= 1; rf_rd <= 5'd4; rf_wd <= alu_result;

## // Change op for next read

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd2; alu_ctrl <= CTRL_SUB;

## state <= S_WR_SUB;

## // NOTE: We stay with rs1/rs2=x1,x2 and just changed alu_ctrl.

## // alu_result will reflect SUB on the NEXT posedge so we need

## // one more cycle before writing. Go back through READ.

## // Actually: since alu_ctrl<=CTRL_SUB takes effect NEXT cycle,

## // alu_result this cycle is still ADD - that's fine, we wrote it.

## // Next cycle state=S_WR_SUB and alu_result will be SUB.

## // This works because alu_ctrl and rs1/rs2 are set together.

## end

## S_WR_SUB: begin

## rf_we <= 1; rf_rd <= 5'd5; rf_wd <= alu_result;

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd2; alu_ctrl <= CTRL_AND;

## state <= S_WR_AND;

## end

## S_WR_AND: begin

## rf_we <= 1; rf_rd <= 5'd6; rf_wd <= alu_result;

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd2; alu_ctrl <= CTRL_OR;

## state <= S_WR_OR;

## end

## S_WR_OR: begin

## rf_we <= 1; rf_rd <= 5'd7; rf_wd <= alu_result;

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd2; alu_ctrl <= CTRL_XOR;

## state <= S_WR_XOR;

## end

## S_WR_XOR: begin

## rf_we <= 1; rf_rd <= 5'd8; rf_wd <= alu_result;

## // Switch to shift ops: rs2=x3 (shift amount=5)

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd3; alu_ctrl <= CTRL_SLL;

## state <= S_READ_SLL;

## end

## // One settle cycle for SLL (rs2 changed to x3)

## S_READ_SLL: begin

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd3; alu_ctrl <= CTRL_SLL;

## state <= S_WR_SLL;

## end

## S_WR_SLL: begin

## rf_we <= 1; rf_rd <= 5'd9; rf_wd <= alu_result;

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd3; alu_ctrl <= CTRL_SRL;

## state <= S_WR_SRL;

## end

## S_WR_SRL: begin

## rf_we <= 1; rf_rd <= 5'd10; rf_wd <= alu_result;

## // BEQ: point both ports at x1 for SUB x1-x1

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd1; alu_ctrl <= CTRL_SUB;

## state <= S_BEQ_READ;

## end

## // Settle cycle for BEQ (rs2 changed to x1)

## S_BEQ_READ: begin

## rf_rs1 <= 5'd1; rf_rs2 <= 5'd1; alu_ctrl <= CTRL_SUB;

## state <= S_BEQ_WR;

## end

## // Write 1 into x11 if Zero flag is set (branch taken)

## S_BEQ_WR: begin

## rf_we <= 1; rf_rd <= 5'd11;

## rf_wd <= alu_zero ? 32'd1 : 32'd0;

## state <= S_DONE;

## end

## S_DONE: begin

## rf_we <= 0;

## state <= S_DONE;   // stay here - no more looping

## end

## default: state <= S_INIT0;

## endcase

## end

## end

## endmodule

## Testbench file for Top-level 
`timescale 1ns / 1ps

## module RF_ALU_FSM_tb;

## reg clk;

## reg rst;

## always #5 clk = ~clk;   // 100 MHz

## reg         rf_we;

## reg  [4:0]  rf_rs1, rf_rs2, rf_rd;

## reg  [31:0] rf_wd;

## wire [31:0] rf_rd1, rf_rd2;

## RegisterFile RF (

## .clk(clk), .rst(rst),

## .WriteEnable(rf_we),

## .rs1(rf_rs1), .rs2(rf_rs2), .rd(rf_rd),

## .WriteData(rf_wd),

## .ReadData1(rf_rd1), .ReadData2(rf_rd2)

## );

## reg  [3:0]  alu_ctrl;

## wire [31:0] alu_result;

## wire        alu_zero;

## alu_32bit ALU (

## .A(rf_rd1),

## .B(rf_rd2),

## .ALUControl(alu_ctrl),

## .ALUResult(alu_result),

## .Zero(alu_zero)

## );

## localparam [3:0]

## IDLE        = 4'd0,

## WRITE_REGS  = 4'd1,   // write x1, x2, x3 with known constants

## ALU_ADD     = 4'd2,   // x4  = x1 ADD x2

## ALU_SUB     = 4'd3,   // x5  = x1 SUB x2

## ALU_AND     = 4'd4,   // x6  = x1 AND x2

## ALU_OR      = 4'd5,   // x7  = x1 OR  x2

## ALU_XOR     = 4'd6,   // x8  = x1 XOR x2

## ALU_SLL     = 4'd7,   // x9  = x1 SLL x3 (shift amount = x3 = 5)

## ALU_SRL     = 4'd8,   // x10 = x1 SRL x3

## BEQ_CHECK   = 4'd9,   // Zero check: x1 SUB x1 â†’ Zero should be 1 â†’ write flag to x11

## RAW_WRITE   = 4'd10,  // write x12 = 0xABCD1234

## RAW_READ    = 4'd11,  // read x12 next cycle and check

## DONE        = 4'd12;

## reg [3:0] state, next_state;

## always @(posedge clk or posedge rst) begin

## if (rst) state <= IDLE;

## else     state <= next_state;

## end

## // ALU control codes matching Lab 6 alu_32bit

## localparam [3:0]

## CTRL_AND = 4'b0000,

## CTRL_OR  = 4'b0001,

## CTRL_ADD = 4'b0010,

## CTRL_SUB = 4'b0110,

## CTRL_XOR = 4'b0100,

## CTRL_SLL = 4'b1000,

## CTRL_SRL = 4'b1001;

## // Tracking results for assertions

## reg [31:0] saved_alu;

## reg        saved_zero;

## always @(posedge clk) begin

## if (rst) begin

## rf_we      <= 0; rf_rd <= 0; rf_wd <= 0;

## rf_rs1     <= 0; rf_rs2 <= 0;

## alu_ctrl   <= CTRL_ADD;

## saved_alu  <= 0; saved_zero <= 0;

## end else begin

## // Defaults: no write, no op change

## rf_we <= 0;

## case (state)

## WRITE_REGS: begin

## // Write x1 = 0x10101010, x2 = 0x01010101, x3 = 5

## // We do all three writes in consecutive cycles via sub-steps.

## // Simple approach: use a shift register for sub-stepping.

## // (For clarity the lab FSM handles one write per state tick.

## //  Three writes: we'll split across WRITE_REGS â†’ two helper

## //  states by reusing the DONE state trick.)

## //

## // In this simplified implementation we write all three using

## // a counter that increments each cycle while we stay here.

## rf_we   <= 1;

## rf_rd   <= 5'd1;

## rf_wd   <= 32'h10101010;

## end

## // We need extra states to write x2, x3.

## // Add them in next_state logic below; the write happens one

## // cycle later thanks to the synchronous RF.

## ALU_ADD: begin

## // x4 = ADD(x1, x2) â€“ set RF read addresses, set ALU op

## rf_rs1   <= 5'd1; rf_rs2 <= 5'd2;

## alu_ctrl <= CTRL_ADD;

## // Capture result and write to x4 in the SAME state

## // (ALU is combinational from RF reads)

## rf_we    <= 1;

## rf_rd    <= 5'd4;

## rf_wd    <= alu_result;

## end

## ALU_SUB: begin

## rf_rs1   <= 5'd1; rf_rs2 <= 5'd2;

## alu_ctrl <= CTRL_SUB;

## rf_we <= 1; rf_rd <= 5'd5; rf_wd <= alu_result;

## end

## ALU_AND: begin

## rf_rs1   <= 5'd1; rf_rs2 <= 5'd2;

## alu_ctrl <= CTRL_AND;

## rf_we <= 1; rf_rd <= 5'd6; rf_wd <= alu_result;

## end

## ALU_OR: begin

## rf_rs1   <= 5'd1; rf_rs2 <= 5'd2;

## alu_ctrl <= CTRL_OR;

## rf_we <= 1; rf_rd <= 5'd7; rf_wd <= alu_result;

## end

## ALU_XOR: begin

## rf_rs1   <= 5'd1; rf_rs2 <= 5'd2;

## alu_ctrl <= CTRL_XOR;

## rf_we <= 1; rf_rd <= 5'd8; rf_wd <= alu_result;

## end

## ALU_SLL: begin

## // SLL: A=x1, B=x3 (shamt = 5)

## rf_rs1   <= 5'd1; rf_rs2 <= 5'd3;

## alu_ctrl <= CTRL_SLL;

## rf_we <= 1; rf_rd <= 5'd9; rf_wd <= alu_result;

## end

## ALU_SRL: begin

## rf_rs1   <= 5'd1; rf_rs2 <= 5'd3;

## alu_ctrl <= CTRL_SRL;

## rf_we <= 1; rf_rd <= 5'd10; rf_wd <= alu_result;

## end

## BEQ_CHECK: begin

## // SUB x1-x1 â†’ Zero should be HIGH â†’ write 1 into x11

## rf_rs1   <= 5'd1; rf_rs2 <= 5'd1;

## alu_ctrl <= CTRL_SUB;

## saved_zero <= alu_zero;

## rf_we <= 1; rf_rd <= 5'd11;

## rf_wd <= alu_zero ? 32'd1 : 32'd0;

## end

## RAW_WRITE: begin

## // Write x12 = 0xABCD1234

## rf_we  <= 1;

## rf_rd  <= 5'd12;

## rf_wd  <= 32'hABCD1234;

## end

## RAW_READ: begin

## // Read x12 â€“ one cycle after write (synchronous RF)

## rf_rs1 <= 5'd12;

## // Data will be on ReadData1 after posedge (checked in assertions)

## end

## DONE: ; // sit here

## endcase

## end

## end

## //

## reg [1:0] wr_cnt;   // sub-step counter for the three initial writes

## always @(posedge clk or posedge rst) begin

## if (rst) wr_cnt <= 0;

## else if (state == WRITE_REGS) wr_cnt <= wr_cnt + 1;

## else wr_cnt <= 0;

## end

## // Parallel write handler for x2, x3 during sub-steps

## // (Separate always block that fires in the same cycle)

## // NB: We override rf_we/rd/wd for sub-steps 1 and 2

## always @(posedge clk) begin

## if (!rst && state == WRITE_REGS) begin

## case (wr_cnt)

## 2'd0: begin rf_we <= 1; rf_rd <= 5'd1; rf_wd <= 32'h10101010; end

## 2'd1: begin rf_we <= 1; rf_rd <= 5'd2; rf_wd <= 32'h01010101; end

## 2'd2: begin rf_we <= 1; rf_rd <= 5'd3; rf_wd <= 32'd5;        end

## default: rf_we <= 0;

## endcase

## end

## end

## always @(*) begin

## case (state)

## IDLE:       next_state = WRITE_REGS;

## WRITE_REGS: next_state = (wr_cnt == 2'd2) ? ALU_ADD : WRITE_REGS;

## ALU_ADD:    next_state = ALU_SUB;

## ALU_SUB:    next_state = ALU_AND;

## ALU_AND:    next_state = ALU_OR;

## ALU_OR:     next_state = ALU_XOR;

## ALU_XOR:    next_state = ALU_SLL;

## ALU_SLL:    next_state = ALU_SRL;

## ALU_SRL:    next_state = BEQ_CHECK;

## BEQ_CHECK:  next_state = RAW_WRITE;

## RAW_WRITE:  next_state = RAW_READ;

## RAW_READ:   next_state = DONE;

## default:    next_state = DONE;

## endcase

## end

## integer pass_count = 0;

## integer fail_count = 0;

## task check;

## input [63:0] label_int;   // just a numeric code for display

## input [31:0] got;

## input [31:0] expected;

## begin

## if (got === expected) begin

## $display("[PASS] State %0d : got=0x%08h  exp=0x%08h", label_int, got, expected);

## pass_count = pass_count + 1;

## end else begin

## $display("[FAIL] State %0d : got=0x%08h  exp=0x%08h", label_int, got, expected);

## fail_count = fail_count + 1;

## end

## end

## endtask

## // Monitor â€“ fire after each rising edge when state changes

## reg [3:0] prev_state;

## always @(posedge clk) begin

## prev_state <= state;

## // One cycle after a result has been committed to the RF, check it

## case (prev_state)

## ALU_ADD: begin

## // x4 should hold ADD result = 0x10101010 + 0x01010101 = 0x11111111

## rf_rs1 = 5'd4; #1;

## check(ALU_ADD, rf_rd1, 32'h11111111);

## end

## ALU_SUB: begin

## // x5 = SUB = 0x10101010 - 0x01010101 = 0x0F0F0F0F

## rf_rs1 = 5'd5; #1;

## check(ALU_SUB, rf_rd1, 32'h0F0F0F0F);

## end

## ALU_AND: begin

## // x6 = AND = 0x10101010 & 0x01010101 = 0x00000000

## rf_rs1 = 5'd6; #1;

## check(ALU_AND, rf_rd1, 32'h00000000);

## end

## ALU_OR: begin

## // x7 = OR = 0x10101010 | 0x01010101 = 0x11111111

## rf_rs1 = 5'd7; #1;

## check(ALU_OR, rf_rd1, 32'h11111111);

## end

## ALU_XOR: begin

## // x8 = XOR = 0x10101010 ^ 0x01010101 = 0x11111111

## rf_rs1 = 5'd8; #1;

## check(ALU_XOR, rf_rd1, 32'h11111111);

## end

## ALU_SLL: begin

## // x9 = SLL x1 by 5 = 0x10101010 << 5 = 0x20202200

## rf_rs1 = 5'd9; #1;

## check(ALU_SLL, rf_rd1, 32'h20202200);

## end

## ALU_SRL: begin

## // x10 = SRL x1 by 5 = 0x10101010 >> 5 = 0x00808080

## rf_rs1 = 5'd10; #1;

## check(ALU_SRL, rf_rd1, 32'h00808080);

## end

## BEQ_CHECK: begin

## // x11 should be 1 (BEQ matched)

## rf_rs1 = 5'd11; #1;

## check(BEQ_CHECK, rf_rd1, 32'd1);

## end

## RAW_READ: begin

## // x12 should be 0xABCD1234 (read-after-write next cycle)

## rf_rs1 = 5'd12; #1;

## check(RAW_READ, rf_rd1, 32'hABCD1234);

## end

## DONE: begin

## if (prev_state != DONE) begin   // only print once

## $display("==================================================");

## $display("RF_ALU_FSM_tb: %0d PASSED, %0d FAILED", pass_count, fail_count);

## $display("==================================================");

## $finish;

## end

## end

## endcase

## end

## //

## initial begin

## clk = 0; rst = 1;

## repeat (4) @(posedge clk);

## rst = 0;

## // FSM runs autonomously; the monitor above does all checking.

## // Safety timeout

## #10000;

## $display("[TIMEOUT] Simulation did not finish within budget");

## $finish;

## end

## endmodule

## Simulation waveforms showing correct operation

# Task 3: FPGA Synthesis and RTL Analysis

Note: The switch interface, button debouncing module, and LED interface must be re used from Lab 5.

## RTL schematic screenshots (register file array, decoder, FSM, ALU connections)

## [ NEXT PAGE ]

## Demonstration of FPGA operation

## [ SHOWN TO MISS IN LAB ]

# Assessment Rubric

Lab 7: Design and FPGA Implementation of the Register File for a Single-Cycle RISC-V Processor

| Name: | Student ID: | Section: |
| --- | --- | --- |

Points Distribution

|  | Task No. | LR 2 (Code) | LR 5 (Results) | LR 11 (Design) |
| --- | --- | --- | --- | --- |
| In-Lab | Task 1 | - | - | /10 |
|  | Task 2 | /20 | /10 | - |
|  | Task 3 | /20 | /20 | - |
| Total Points |  | /40 | /30 | /10 |
| CLO Mapped |  | CLO 1 |  |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 1 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 1 | 100 |  |
| Total | 100 |  |

For description of different levels of the mapped rubrics, please refer to the Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics provided here.